# Module -- AbsHwPosScom

This module determines the handwheel position representing the angle of the output shaft without using a  physical column position sensor.

The function uses the Motor Position to provide an estimate of the handwheel position. In addition this function also uses the absolute hw position transmitted on the serial communication interface to provide and estimate of the HwPos. value.

# High-Level Description

(Description must be within 8-10 lines.)

# Figures

## Diagram – Function Data Sharing

### Diagram – Function (Name)

# Variable Data Dictionary

For details on module input / output variable, refer to the Data Dictionary for the application.  Input / output variable names are listed here for reference.

(Note: Full variable names required in table.)

(Note: All global variables including End Of Line data used should be shown here)

| Module Inputs | Module Outputs |  |
| --- | --- | --- |
| CRFCumMtrPos_Deg_f32 |  | HwPosAuthority_Uls_f32 |
| CRFMtrTrqCmd_MtrNm_f32 |  | HwPos_HwDeg_f32 |
| CRFMtrVel_MtrRadpS_f32 |  |  |
| DiagStsHwPosDis_Cnt_lgc |  |  |
| HwTrq_HwNm_f32 |  |  |
| SComInpHwAValid_Cnt_lgc |  |  |
| SComInpHwA_HwDeg_f32 |  |  |
| VehicleSpeedValid_Cnt_lgc |  |  |
| VehicleSpeed_Kph_f32 |  |  |
| KinIntDiagSrlComSvcDft_Cnt_lgc |  |  |

## Module Internal Variables

This section identifies the name, range and resolutions for module specific data created by this module.  If there are no range restrictions on the variable, the term “FULL” is placed into the table for legal range.

| Variable Name | Resolution | Legal Range (min) | Legal Range (max) | Software Segment |
| --- | --- | --- | --- | --- |
| HwPFilter1En_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| HwPFilter2En_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| VehDynTrgtAngle_HwDeg_M_f32 | Single precision floating point | -1575164.8 | 1575164.8 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| VehDynTrgtAuth_Uls_M_f32 | Single precision floating point | 0 | 1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPHighSpdAC_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| PinTrqFiltSV_HwNm_M_Str | N/A | N/A | N/A | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_UNPECIFIED |
| PinTrqFiltSV_HwNm_M_Str .K_Uls_f32 | Single precision floating point | 8.04215E-05 | 0.039414597 |  |
| PinTrqFiltSV_HwNm_M_Str.SV_Uls_f32 | Single precision floating point | -318 | 318 |  |
| HwPMtrVelT1Cal_MtrRadpS_M_f32 | Single precision floating point | 0 | 700 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPVSpdT1Cal_Kph_M_f32 | Single precision floating point | 0 | 255 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPPinTrqT1Cal_HwNm_M_f32 | Single precision floating point | 0 | 10 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPTimer1Max_ms_M_u16 | 1 | 0 | 60000 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_16 |
| HwPWindowCal_HwDeg_M_f32 | Single precision floating point | 0 | 100 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPFilter1Kn_Hz_M_f32 | Single precision floating point | 0.0002 | 0.1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPFilter2Kn_Hz_M_f32 | Single precision floating point | 0.0002 | 0.1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| AutoCntrTrgtAuth_Uls_M_f32 | Single precision floating point | 0 | 1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPTimer2Max_ms_M_u16 | 1 | 0 | 60000 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_16 |
| HwPAutoCenter_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| HwPFilter1Init_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| HwPFilter2Init_Cnt_M_lgc | BOOLEAN | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| HwPFilter1Prev_HwDeg_M_f32 | Single Precision Floating point | -787558.4 | 787606.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPFilter1Out_HwDeg_M_f32 | Single Precision Floating point | -787558.4 | 787606.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPFilter1SV_HwDeg_M_Str | N/A | N/A | N/A | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_UNPECIFIED |
| HwPFilter1SV_HwDeg_M_Str.K_Uls_f32 | Single Precision Floating point | 8.04215E-05 | 0.039414597 |  |
| HwPFilter1SV_HwDeg_M_Str.SV_Uls_f32 | Single Precision Floating point | -787558.4 | 787606.4 |  |
| HwPFilter2Out_HwDeg_M_f32 | Single Precision Floating point | -787558.4 | 787606.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPFilter2SV_HwDeg_M_Str | N/A | N/A | N/A | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_UNPECIFIED |
| HwPFilter2SV_HwDeg_M_Str .K_Uls_f32 | Single Precision Floating point | 8.04215E-05 | 0.039414597 |  |
| HwPFilter2SV_HwDeg_M_Str *SV_Uls_f32 | Single Precision Floating point | -787558.4 | 787606.4 |  |
| HwPTimer1_ms_M_u32 | 1 | FULL | FULL | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPTimer2_ms_M_u32 | 1 | FULL | FULL | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| HwPTimer4_ms_M_u32 | 1 | FULL | FULL | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| ScaledMtrPos_HwDeg_M_f32 | Float32 | -787558.4 | 787606.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| MtrComplErr_HwDeg_M_f32 | Float32 | -1126.4 | 1126.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| TempHwPos_HwDeg_M_f32 | Float32 | -1575164.8 | 1575164.8 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| RelHwPos_HwDeg_M_f32 | Float32 | -787558.4 | 787606.4 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| FiltPinionTrq_HwNm_M_f32 | single precision floating point | -318 | 318 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| FinHwPosLPFOp_HwDeg_M_f32 | single precision floating point | -1620 | 1620 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| FinHwAuth_Uls_M_f32 | single precision floating point | 0 | 1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| FinHwPosFiltSV_HwDeg_M_Str | N/A | N/A | N/A | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_UNPECIFIED |
| FinHwPosFiltSV_HwDeg_M_Str.K_Uls_f32 | single precision floating point | 0 | 0.999984637 |  |
| FinHwPosFiltSV_HwDeg_M_Str.SV_Uls_f32 | single precision floating point | -1575164.8 | 1575164.8 |  |
| RelToSComOffset_HwDeg_M_f32 | Float32 | -789654 | 789606.3 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| StabilitySyncTimer_mS_M_u32 | UINT32 | FULL | FULL | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| NewScomInpHwA_HwDeg_M_f32 | Single precison floating point | -1578812.8 | 1578812.738 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| ScomHwPosAuth_Uls_M_f32 | Single precison floating point | 0 | 1 | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_32 |
| VdynToScomErrorThy_HwDeg_M_f32 | Single precison floating point | -1600 | 1600 | ABSHWPOSSCOM_START_SEC_VAR_SAVED_ZONEH_32 |
| HwPosSrvSetToZero_Cnt_M_lgc | n/a | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |
| RelToScomOfstFound_Cnt_M_lgc | n/a | FALSE | TRUE | ABSHWPOSSCOM_START_SEC_VAR_CLEARED_BOOLEAN |

### User defined typedef definition/declaration

This section documents any user types uniquely used for the module.

| Typedef Name | Element Name | User Defined Type | Legal Range (min) | Legal Range (max) |
| --- | --- | --- | --- | --- |
| (Name given for the user defined typdef of type struct/union) (Variable name qualified similar to all other variables) | (Variable name qualified similar to all other variables) | as other variables |  |  |
|  | (Variable name qualified similar to all other variables) |  |  |  |

# Constant Data Dictionary

## Calibration Constants

This section lists the calibrations used by the module.  For details on calibration constants, refer to the Data Dictionary for the application.

Constant Name

k_AutoCtrMtrVelT1L_MtrRadpS_f32

k_AutoCtrVehSpdT1L_Kph_f32 k_AutoCtrPinTrqT1L_HwNm_f32 k_HwPTimer1MaxL_ms_u16 k_AutoCtrWindowL_HwDeg_f32 k_AutoCtrFiltOneCoeffkL_Hz_f32 k_AutoCtrFiltTwoCoeffkL_Hz_f32 k_VDAuthL_Uls_f32 k_AutoCtrTimer2LSpd_ms_u16 k_InvGearRatio_Uls_f32 k_AutoCtrPinTrqLPFCoeffK_Hz_f32 k_MtrVelTmr4H_MtrRadpS_f32 k_VehSpdTmr4H_kph_f32 k_PinTrqTmr4H_HwNm_f32 k_Tmr4CntrWindowH_HwDeg_f32 k_AutoCtrTimer4H_ms_u16 k_HwPMtrVelT1H_MtrRadpS_f32 k_HwPVehSpdT1H_Kph_f32 k_HwPPinionTrqT1H_HwNm_f32 k_HwPTimer1MaxH_ms_u16 k_HwPWindowCalH_HwDeg_f32 k_HwPFilter1KnH_Hz_f32 k_HwPFilter2KnH_Hz_f32 k_HiSpdAcAuth_Uls_f32 k_HwPTmr2MaxHiSpd_ms_u16 t_MtrPosComplTblX_MtrNm_u8p8[6] t_MtrPosComplTblY_HwDegpMtrNm_u6p10[6] k_RbstPosLPFKn_Hz_f32 k_KinmIntDiagMaxRackLmt_HwDeg_f32 k_HwPosOpLPFError_HwDeg_f32 k_HwPosAuthorityStep_Uls_f32

k_ScomHwPosAuth_Uls_f32

k_HwPosVehStabilityTime_mS_u16

k_HwPosVehStabilityTrqCmd_MtrNm_f32

k_HwPosVehStabilityHwTrq_HwNm_f32

k_HwPosVehStabilityMtrVel_MtrRadpS_f32

k_VdynToScomMaxErr_HwDeg_f32

## Program(fixed) Constants

### Embedded Constants

All embedded constants whose values are provided in Eng units will be evaluated to the equivalent counts by using the FPM_InitFixedPoint_m() macro within the #define statement.

#### Local

| Constant Name | Resolution | Units | Value |
| --- | --- | --- | --- |
| D_ABSPOSLOLMT_HWDEG_F32 | Single precision floating point | HWDEG | -1620 |
| D_ABSPOSHILMT_HWDEG_F32 | Single precision floating point | HWDEG | 1620 |
| D_10MS_SEC_F32 | Single precision floating point | SEC | 0.010 |

#### Global

This section lists the global constants used by the module.  For details on global constants, refer to the Data Dictionary for the application.

Constant Name

D_2MS_SEC_F32

D_ONE_ULS_F32

### Module specific Lookup Tables Constants

(This is for lookup tables (arrays) with fixed values, same name as other tables)

| Constant Name | Resolution | Value | Software Segment |
| --- | --- | --- | --- |
| None |  |  |  |

# Functions/Macros used by the Sub-Modules

## Library Functions / Macros

The library and functions / Macros that are called by the various sub modules are identified below,

- FPM_FloatToFixed_m

- LPF_SvUpdate_s16InFixKTrunc_m

- LPF_OpUpdate_s16InFixKTrunc_m

- Limit_m

- FPM_FixedToFloat_m

- Min_m

- Max_m

- LPF_SvInit_s16InFixKTrunc_m

## Data Hiding Functions

- Rte_Call_NxtrDiagMgr_SetNTCStatus

## Global Functions/Macros Defined by this Module

### Global Function #1

| Function Name | (Exact name used) | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | (if none, write None) |  |  |  |
|  | (Insert more rows for additional passed arguments) |  |  |  |
| Return Value | (if no value returned, write N/A) |  |  |  |

#### Description

(Place flowchart/design for local function)

## Local Functions/Macros Used by this MDD only

### Local Function #1

| Function Name | HwPosSLFilter1 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | MtrPos_HwDeg_T_f32 | float32 | -787558.4 | 787606.4 |
|  | FilterEn | boolean | FALSE | TRUE |
| Return Value | (if no value returned, write N/A) |  |  |  |

#### Description

(Place flowchart/design for local function)

### Local Function #2

| Function Name | HwPosSLFilter2 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | MtrPos_HwDeg_T_f32 | float32 | -787558.4 | 787606.4 |
|  | FilterEn | boolean | FALSE | TRUE |
| Return Value | (if no value returned, write N/A) |  |  |  |

#### Description

(Place flowchart/design for local function)

### Local Function #3

| Function Name | Filter1Enable | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | CRFMtrVel_MtrRadpS_T_f32 | float32 | -1118 | 1118 |
|  | VehSpd_Kph_T_f32 | float32 | 0 | 512 |
|  | VSpdValid _Cnt_T_lgc | boolean | FALSE | TRUE |
|  | PinionTrq_HwNm_T_f32 | float32 | -318 | 318 |
| Return Value | HwPFilter1En_Cnt_T_lgc | boolean | FALSE | TRUE |

#### Description

(Place flowchart/design for local function)

### Local Function #4

| Function Name | Filter2Enable | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | MtrPos_HwDeg_T_f32 | float32 | -787558.4 | 787606.4 |
|  | HwPFilter1En_Cnt_T_lgc | boolean | FALSE | TRUE |
| Return Value | HwPFilter2En_Cnt_T_lgc | boolean | FALSE | TRUE |

#### Description

(Place flowchart/design for local function)

### Local Function #5

| Function Name | CalSwitchTimer | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | CRFMtrVel_MtrRadpS_T_f32 | float32 | -1118 | 1118 |
|  | VehSpd_Kph_T_f32 | float32 | 0 | 512 |
|  | VSpdValid _Cnt_T_lgc | boolean | FALSE | TRUE |
|  | PinionTrq_HwNm_T_f32 | float32 | -318 | 318 |
|  | HwPFilter1En_Cnt_T_lgc | boolean | FALSE | TRUE |
|  | HwPFilter2En_Cnt_T_lgc | boolean | FALSE | TRUE |
| Return Value | None |  |  |  |

#### Description

(Place flowchart/design for local function)

### Local Function #6

| Function Name | ScomHwPosAuth | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | None |  |  |  |
| Return Value | None |  |  |  |

#### Description

# Software Module Implementation

## Runtime Environment (RTE) Initial Values

This section lists the initial values of data written by this module but controlled by the RTE. After RTE initialization, the data in this table will contain these values.

| Data | Value |
| --- | --- |
| CRFCumMtrPos_Deg_f32 | 0 |
| CRFMtrTrqCmd_MtrNm_f32 | 0 |
| CRFMtrVel_MtrRadpS_f32 | 0 |
| DiagStsHwPosDis_Cnt_lgc | FALSE |
| HwTrq_HwNm_f32 | 0 |
| SComInpHwAValid_Cnt_lgc | FALSE |
| SComInpHwA_HwDeg_f32 | 0 |
| VehicleSpeedValid_Cnt_lgc | FALSE |
| VehicleSpeed_Kph_f32 | 0 |
| KinIntDiagSrlComSvcDft_Cnt_lgc | FALSE |
| HwPosAuthority_Uls_f32 | 0 |
| HwPos_HwDeg_f32 | 0 |

## Initialization Functions

(Note: For multiple init functions, insert new headers at the “Header 2” level – subset of “5.1 Initialization Functions” and follow the same sub-section design shown below)

### Init: AbsHwPosScom_Init1

### Design Rationale

(Add design rationale specifically related to this FUNCTION.  If none required, place the text “None”)

#### Module Outputs

(Initialize all module outputs in this section)

## Periodic Functions

### Per: AbsHwPosScom_Per1

#### Design Rationale

#### Program Flow Start

#### Rte_Call_AbsHwPosScom_Per1_CP0_CheckpointReached()Store Module Inputs to Local copies

#### Processing

#### Store Local copy of outputs into Module Outputs

#### Program Flow End

Rte_Call_AbsHwPosScom_Per1_CP1_CheckpointReached()

### Per: AbsHwPosScom_Per2

#### Design Rationale

(Add design rationale specifically related to this FUNCTION.  If none required, place the text “None”)

#### Program Flow Start

#### Rte_Call_AbsHwPosScom_Per2_CP0_CheckpointReached()Store Module Inputs to Local copies

#### Program Flow End

Rte_Call_AbsHwPosScom_Per2_CP1_CheckpointReached()

### Per: AbsHwPosScom_Per3

#### Design Rationale

(Add design rationale specifically related to this FUNCTION.  If none required, place the text “None”)

#### Program Flow Start

#### Rte_Call_AbsHwPosScom_Per3_CP0_CheckpointReached()Store Module Inputs to Local copies

#### Program Flow End

Rte_Call_AbsHwPosScom_Per3_CP1_CheckpointReached()

## Fault Recovery Functions

## Shutdown Functions

## Interrupt Functions

## Serial Communication Functions

### AbsHwPosScom_Scom_HwPosSrvSetToZero

| Function Name | ### AbsHwPosScom_Scom_HwPosSrvSetToZero | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Par | Boolean | False | True |
| Return Value | None |  |  |  |

#### Design Rationale

#### Program Flow Start

#### Store Module Inputs to Local copies

N/A

#### Processing

HwPosSrvSetToZero_Cnt_M_lgc = Par

#### Store Local Copy Of Outputs into Module Outputs

N/A

#### Program Flow End

N/A

### AbsHwPosScom_Scom_HwPosSrvRead

| Function Name | ### AbsHwPosScom_Scom_ HwPosSrvRead | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Par | Ptr to Boolean | False | True |
| Return Value | None |  |  |  |

#### Design Rationale

#### Program Flow Start

#### Store Module Inputs to Local copies

N/A

#### Processing

*Par = HwPosSrvSetToZero_Cnt_M_lgc

#### Store Local Copy Of Outputs into Module Outputs

N/A

#### Program Flow End

N/A

# Execution Requirements

## Execution Sequence of the Module

(Describe in words relevant details about the execution sequence of the different sub modules.)

## Execution Rates for sub-modules called by the Scheduler

This table serves as reference for the Scheduler design

| Function Name | Calling Frequency | System State(s) in which the function is called |
| --- | --- | --- |
| AbsHwPosScom_Init1 | Once at startup | ColdInit |
| AbsHwPosScom_Per1 | 2ms | All states except ColdInit |
| AbsHwPosScom_Per2 | 10ms | All states except Cold Init |
| AbsHwPosScom_Per3 | 10ms | All states except Cold Init |

## Execution Requirements for Serial Communication Functions

| Function Name | Sub-Module called by (Serial Comm Function Name) |
| --- | --- |
| AbsHwPosScom_Scom_HwPosSrvRead |  |
| AbsHwPosScom_Scom_HwPosSrvSetToZero |  |

# Memory Map Definition Requirements

## Sub Modules (Functions)

This table identifies the software segments for functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| AbsHwPosScom_Init1 | RTE_AP_ABSHWPOSSCOM_APPL_CODE |
| AbsHwPosScom_Per1 | RTE_AP_ABSHWPOSSCOM_APPL_CODE |
| AbsHwPosScom_Per2 | RTE_AP_ABSHWPOSSCOM_APPL_CODE |
| AbsHwPosScom_Per3 | RTE_AP_ABSHWPOSSCOM_APPL_CODE |

## Local Functions

This table identifies the software segments for local functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| HwPosSLFilter1 | AP_ ABSHWPOSSCOM_CODE |
| HwPosSLFilter2 | AP_ ABSHWPOSSCOM_CODE |
| Filter1Enable | AP_ ABSHWPOSSCOM_CODE |
| Filter2Enable | AP_ ABSHWPOSSCOM_CODE |
| CalSwitchTimer | AP_ ABSHWPOSSCOM_CODE |

# Known Issues / Limitations With Design

# Note1: Deviation from FDD: Cal k_MtrToHw_Res_Conversion is not used for Scaled_MtrPos calculations in this software. Refer FDD- 05D section 5.1.2.2.(This cal was for a fixedtype implementation).
Revision Control Log

| Item # | Rev # | Change Description | Date | Author Initials |
| --- | --- | --- | --- | --- |
| 1 | 1.0 | Initial AutoSAR Release | 24-April-12 | NRAR |
| 2 | 2.0 | Text format changes | 25-April-12 | NRAR |
| 3 | 3.0 | HandWheelPosition range is changed from +/-1440.11 to +/-1620HwDeg | 26-April-12 | NRAR |
| 4 | 4.0 | Anom 3257 Fix | 4-May-12 | NRAR |
| 5 | 5.0 | FDD 5D updates as per Ver002 | 27-June-12 | NRAR |
| 6 | 6.0 | FDD 5D updates as per ver 003,Ver002 and Anom 3382 is ignored as now s/w is implemented in floating type. | 05-July-12 | NRAR |
| 6 | 6 | - Version of thisMDD is still lshown as 6 to match up with Synergy version - In per1, Typo fix for global inputs | 19-July-12 | NRAR |
| 7 | 7.0 | FDD 5D updates per Ver006 | 10-Aug-12 | BWL |
| 8 | 8.0 | Update per UTP review. | 5-Sept-12 | BWL |
| 9 | 9.0 | Added checkpoints and memmap software segment is updated for static variables | 26-Sep-12 | Selva |
| 10 | 10.0 | Changed trigger rates for Per2 and Per3 to conform to new standard | 24-Oct-12 | BWL |