# Module – Motor Position 2

# High-Level Description

This module implements the learning and diagnostic portions of MtrPos, as well as part of the motor position determination sub function.

# Figures

## Component Diagram

# Variable Data Dictionary

For details on module input / output variable, refer to the Data Dictionary for the application.  Input / output variable names are listed here for reference.

| Module Inputs | Module Outputs |  |
| --- | --- | --- |
| MotorVelMRF_MtrRadpS_f32 |  | CorrectedMtrPos_Rev_f32 |
|  |  | CosTheta1_Volt_f32 |
|  |  | MechMtrPos_Rev_f32 |
|  |  | SinTheta1_Volt_f32 |
|  |  | SysCCorrectedMtrPos_Rev_f32 |
|  |  | SysCMechMtrPos_Rev_f32 |

## Module Internal Variables

This section identifies the name, range and resolutions for module specific data created by this module.  If there are no range restrictions on the variable, the term “FULL” is placed into the table for legal range.

| Variable Name | Resolution | Legal Range (min) | Legal Range (max) | Software Segment |
| --- | --- | --- | --- | --- |
| MtrPos2_ValidityFltAcc_Cnt_M_u16 | 1 | 0 | 65535 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos2_CorrFltAcc_Cnt_M_u16 | 1 | 0 | 65535 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos2_PrevCorrectedMtrPos_Rev_M_f32 | Single Precision Float | 0 | 0.99998 | MTRPOS2_START_SEC_VAR_CLEARED_32 |
| MtrPos2_PrevSin1RTOffset_Volts_M_f32 | Single Precision Float | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_PrevCos1RTOffset_Volts_M_f32 | Single Precision Float | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Sin1OffsetCorr_Volts_M_f32 | Single Precision Float | -0.5 | 0.5 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Cos1OffsetCorr_Volts_M_f32 | Single Precision Float | -0.5 | 0.5 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Sin1GainCorr_Uls_M_f32 | Single Precision Float | 0.8 | 1.2 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Cos1GainCorr_Uls_M_f32 | Single Precision Float | 0.8 | 1.2 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_CosSin1NomRatio_Uls_M_f32 | Single Precision Float | 1 | 10 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_SinCos1NomRatio_Uls_M_f32 | Single Precision Float | 1 | 10 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_RTToNomHighLmt_Uls_M_f32 | Single Precision Float | 1 | 1.02 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_RTToNomLowLmt_Uls_M_f32 | Single Precision Float | 0.98 | 1 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Cos1RTAmpRec_Uls_M_f32 | Single Precision Float | 0.2 | 3.3 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Cos1RTOffset_Volt_M_f32 | Single Precision Float | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Sin1RTAmpRec_Uls_M_f32 | Single Precision Float | 0.2 | 3.3 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Sin1RTOffset_Volt_M_f32 | Single Precision Float | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_NOINIT_32 |
| MtrPos2_Cos1MaxSV_Volts_M_s2p29 | 2-29 | 0.25 | 2.5 | MTRPOS2_START_SEC_VAR_SAVED_ZONEH_32 |
| MtrPos2_Cos1MinSV_Volts_M_s2p29 | 2-29 | -0.25 | -2.5 | MTRPOS2_START_SEC_VAR_SAVED_ZONEH_32 |
| MtrPos2_Sin1MaxSV_Volts_M_s2p29 | 2-29 | 0.25 | 2.5 | MTRPOS2_START_SEC_VAR_SAVED_ZONEH_32 |
| MtrPos2_Sin1MinSV_Volts_M_s2p29 | 2-29 | -0.25 | -2.5 | MTRPOS2_START_SEC_VAR_SAVED_ZONEH_32 |
| MtrPos2_ValidErr_VoltsSqrd_D_f32 | Single Precision Float | 0 | 4.5 | MTRPOS2_START_SEC_VAR_CLEARED_32 |
| MtrPos2_LearnedParamBfr_Cnt_M_u08 | 1 | 0 | 1 | MTRPOS2_START_SEC_VAR_CLEARED_8 |
| MtrPos2_Sin1RTOffset_Volts_M_u3p13[D_MTRPOSDBLBUFFSZ_CNT_U08] | 2^-13 | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos2_Cos1RTOffset_Volts_M_u3p13[D_MTRPOSDBLBUFFSZ_CNT_U08] | 2^-13 | 1.7 | 3.8 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos2_Sin1RTAmpRec_Uls_M_u3p13[D_MTRPOSDBLBUFFSZ_CNT_U08] | 2^-13 | 0.2 | 3.3 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos_Cos1RTAmpRec_Uls_M_u3p13[D_MTRPOSDBLBUFFSZ_CNT_U08] | 2^-13 | 0.2 | 3.3 | MTRPOS2_START_SEC_VAR_CLEARED_16 |
| MtrPos2_ErrorTerm_Rev_D_f32 | Single Precision Float | 0 | 0.5 | MTRPOS2_START_SEC_VAR_CLEARED_32 |

### User defined typedef definition/declaration

This section documents any user types uniquely used for the module.

| Typedef Name | Element Name | User Defined Type | Legal Range (min) | Legal Range (max) |
| --- | --- | --- | --- | --- |
| MtrPosCal_DataType | BEMFCal_Rev_u0p16 R_BEMFCal_Rev_u0p16 Sin1Offset_Volts_u3p13 Sin1AmpRec_Uls_u3p13 Cos1Offset_Volts_u3p13 Cos1AmpRec_Uls_u3p13 SinDelta1_Uls_s2p13 CosDelta1Rec_Uls_u3p13 Sin1OffCorr_Volts_s2p13 Sin1GainCorr_Uls_u1p15 Cos1OffCorr_Volts_s2p13 Cos1GainCorr_Uls_u1p15 SinHarTbl_Cnt_sm6p13[144] CosHarTbl_Cnt_sm6p13[144] |  | 0 0 2.2 0.25 2.2 0.25 -0.0174524 0.99985 -0.5 0.8 -0.5 0.8 -0.01551 -0.01551 | 1 1 2.8 2.5 2.8 2.5 0.0174524 1 0.5 1.2 0.5 1.2 0.01551 0.01551 |

*In FDD 06B, Ver 6, range for SinHarTbl and CosHarTbl is given as -1 to 1 which is incorrect as it’s a 8bit signal. FDD needs to be updated. Informed to FDD Owner.

# Constant Data Dictionary

## Calibration Constants

This section lists the calibrations used by the module.  For details on calibration constants, refer to the Data Dictionary for the application.

Constant Name

k_RTOffVelThr_MtrRadpS_f32

k_RTFiltEnThresh_Uls_f32

k_RTOffFiltKn_Cnt_u16

k_RTOffsetLmt_Volts_f32

k_AmpRecVarLmt_Uls_f32

k_RTToNomRatioVar_Uls_f32

k_CorrelationError_Rev_f32

k_MtrPosCorrDiag_Cnt_str

k_ValMinError_VoltsSqrd_f32

k_ValMaxError_VoltsSqrd_f32

k_MtrPosValDiag_Cnt_str

## Program(fixed) Constants

### Embedded Constants

All embedded constants whose values are provided in Eng units will be evaluated to the equivalent counts by using the FPM_InitFixedPoint_m() macro within the #define statement.

#### Local

| Constant Name | Resolution | Units | Value |
| --- | --- | --- | --- |
| D_DEGPERREV_ULS_F32 | Single Precision Float | Unitless | 360 |
| D_OFFSETCORRTHRESH_VOLTS_F32 | Single Precision Float | Volts | 0.02 |
| D_AMPCORRTHRESH_ULS_F32 | Single Precision Float | Unitless | 0.02 |
| D_MAXAMPLITUDE_VOLTS_F32 | Single Precision Float | Volts | 2.5 |
| D_MINAMPLITUDE_VOLTS_F32 | Single Precision Float | Volts | 0.25 |
| D_HALF_ULS_F32 | Single Precision Float | Unitless | 0.5 |
| D_TWO_ULS_F32 | Single Precision Float | Unitless | 2 |
| D_WORDMASK_CNT_U16 | 1 | Counts | 0xFFFF |

#### Global

This section lists the global constants used by the module.  For details on global constants, refer to the Data Dictionary for the application.

Constant Name

D_ZERO_ULS_F32

D_ONE_ULS_F32

D_2PI_ULS_F32

D_ZERO_CNT_U16

### Module specific Lookup Tables Constants

| Constant Name | Resolution | Value | Software Segment |
| --- | --- | --- | --- |
| None |  |  |  |

# Functions/Macros used by the Sub-Modules

## Library Functions / Macros

The library and functions / Macros that are called by the various sub modules are identified below,

- FPM_FixedToFloat_m

- FPM_FloatToFixed_m

- Abs_f32_m

- sinf

- cosf

- DiagPStep_m

- DiagNStep_m

- DiagFailed_m

- LPF_SvUpdate_s16InFixKTrunc_m

- Limit_m

## Data Hiding Functions

- Rte_Pim_MtrPosSnsr_EOLData

- Rte_Call_NxtrDiagMgr_GetNTCFailed

## Global Functions/Macros Defined by this Module

None

## Local Functions/Macros Used by this MDD only

### Offset Initialization

| Function Name | OffsetInit | Type | Min | Max | UTP Tol. |
| --- | --- | --- | --- | --- | --- |
| Arguments Passed | EOLAmpRec_Uls_T_f32 | float32 | 0.25 | 2.5 |  |
|  | EOLOffset_Volts_T_f32 | float32 | 2.2 | 2.8 |  |
|  | StateVarMaxPtr_Volts_T_s2p29 | sint32 * | 0.25 | 2.5 | 2-13 |
|  | StateVarMinPtr_Volts_T_s2p29 | sint32 * | -2.5 | -0.25 | 2-13 |
| Return Value | RTOffset_Volts_T_f32 | float32 | 2.2 | 2.8 | 2-13 |

#### Note: In FDD, Range on StoredMax_volts, Stored_Mins are given as 0.25 to 2.5 and -2.5 to -0.25 respt. And in this local function these signals are compared to D_MinAmplitude, D_MaxAmplitude whose const value is 0.25 or 2.5v. 100%coverage for the below IF statement is not achieved with provided ranges. FDD should correct range and is informed to FDD owner. For UnitTesting purpose, special vectors are taken to satisfy IF condition

#### Description

### Run Time Filter

| Function Name | RunTimeFilter | Type | Min | Max | UTP Tol. |
| --- | --- | --- | --- | --- | --- |
| Arguments Passed | SigEst_Uls_T_f32 | float32 | -1 | 1 |  |
|  | SigCorr_Volts_T_f32 | float32 | -3.8 | 3.3 |  |
|  | OffsetDelta_Volts_T_f32 | float32 | -1.1 | 1.6 |  |
|  | StateVarPtr_Volts_T_s2p29 | s2p29_T | -2.5 | 2.5 |  |
| Return Value | Output_Volts_T_f32 | float32 | -2.5 | 2.5 | 2-13 |

#### Description

### Calculate Offset from Min/Max

| Function Name | CalcOffsetFromMinMax | Type | Min | Max | UTP Tol. |
| --- | --- | --- | --- | --- | --- |
| Arguments Passed | Min_Volts_T_f32 | float32 | -0.25 | -2.5 |  |
|  | Max_Volts_T_f32 | float32 | 0.25 | 2.5 |  |
|  | EOLOffset_Volts_T_f32 | float32 | 2.2 | 2.8 |  |
| Return Value | RTOffset_Volts_T_f32 | float32 | 1.7 | 3.8 | 2-13 |

#### Description

### Calculate Sine AmpRec

| Function Name | CalcSinAmpRec | Type | Min | Max | UTP Tol. |
| --- | --- | --- | --- | --- | --- |
| Arguments Passed | TotSinAmp_Volts_T_f32 | float32 | 0.5 | 5 |  |
|  | EOLSinAmpRec_Uls_T_f32 | float32 | 0.25 | 2.5 |  |
| Return Value | SinRTAmpRec_Uls_T_f32 | float32 | 2.2 | 3.3 | 2-13 |

#### Description

### Calculate Cosine AmpRec

| Function Name | CalcCosAmpRec | Type | Min | Max | UTP Tol. |
| --- | --- | --- | --- | --- | --- |
| Arguments Passed | TotSinAmp_Volts_T_f32 | float32 | 0.5 | 5 |  |
|  | TotCosAmp_Volts_T_f32 | float32 | 0.5 | 5 |  |
|  | SinRTAmpRec_Uls_T_f32 | float32 | 0.2 | 3.3 |  |
|  | SinCosNomRatio_Uls_T_f32 | float32 | 1 | 10 |  |
|  | CosSinNomRatio_Uls_T_f32 | float32 | 1 | 10 |  |
|  | RTToNomLowLmt_Uls_T_f32 | float32 | 0.98 | 1 |  |
|  | RTToNomHighLmt_Uls_T_f32 | float32 | 1 | 1.02 |  |
| Return Value | CosRTAmpRec_Uls_T_f32 | float32 | 1.96. | 33.66 | 2-13 |

#### Description

# Software Module Implementation

## Runtime Environment (RTE) Initial Values

This section lists the initial values of data written by this module but controlled by the RTE. After RTE initialization, the data in this table will contain these values.

| Data | Value |
| --- | --- |
| Rte_InitValue_CorrectedMtrPos_Rev_f32 | 0 |
| Rte_InitValue_Cos1RTAmpRec_Uls_f32 | 0 |
| Rte_InitValue_Cos1RTOffset_Volt_f32 | 0 |
| Rte_InitValue_DiagCorrectedMtrPos_Rev_f32 | 0 |
| Rte_InitValue_MechMtrPos_Rev_f32 | 0 |
| Rte_InitValue_MotorVelMRF_MtrRadpS_f32 | 0 |
| Rte_InitValue_Sin1RTOffset_Volt_f32 | 0 |

## Initialization Functions

### Init: MtrPos2_Init1

#### Design Rationale

None

#### Calculate Ratios

#### Check Correction Terms

#### Calculate Limits and Initialize RT Offsets

#### Initialize AmpRec

## Periodic Functions

### Per: MtrPos2_Per1

#### Design Rationale

This function should be run before MtrPos3_Per1, as the comparisons (in both the main and diverse paths) on the generated signals assume that this order will be followed.

#### Program Flow Start

Rte_Call_MtrPos2_Per1_CP0_CheckpointReached()

#### Store Module Inputs to Local copies

CDD_Read_MtrPos_ActvDblBuf_Cnt_u08(&IdleDblBuf_Cnt_T_u08)

IdleDblBuf_Cnt_T_u08 = (IdleDblBuf_Cnt_T_u08 & 1u)^1u

CorrectedMtrPos_Rev_T_f32 = FPM_FixedToFloat_m(MtrPos_CorrectedMtrPos_Rev_M_u0p16[IdleDblBuf_Cnt_T_u08], u0p16_T)

Cos1Scaled_Volt_T_f32 = FPM_FixedToFloat_m(MtrPos_Cos1Scaled_Volts_M_u3p13[IdleDblBuf_Cnt_T_u08], u3p13_T)

DiagCorrectedMtrPos_Rev_T_f32 = MtrPos3_DiagCorrectedMtrPos_Rev_M_f32

MechMtrPos_Rev_T_f32 = _FixedToFloat_m(MtrPos_MechMtrPos_Rev_M_u0p16[IdleDblBuf_Cnt_T_u08], u0p16_T)

MotorVelMRF_MtrRadpS_T_f32 = Rte_IRead_MtrPos2_Per1_MotorVelMRF_MtrRadpS_f32()

Sin1Scaled_Volt_T_f32 = FPM_FixedToFloat_m(MtrPos_Sin1Scaled_Volts_M_u3p13[IdleDblBuf_Cnt_T_u08], u3p13_T)

Rte_Call_NxtrDiagMgr_GetNTCFailed(NTC_Num_PriMSB_SinCosCorr, &MtrPosFault1_Cnt_T_lgc)

Rte_Call_NxtrDiagMgr_GetNTCFailed(NTC_Num_PriVsSec_SinCosCorr, &MtrPosFault2_Cnt_T_lgc)

Sin1Offset_Volts_T_f32 = FPM_FixedToFloat_m(MtrPos_EOLDataPtr_Cnt_M_Str->Sin1Offset_Volts_u3p13, u3p13_T)

Cos1Offset_Volts_T_f32 = FPM_FixedToFloat_m(MtrPos_EOLDataPtr_Cnt_M_Str->Cos1Offset_Volts_u3p13, u3p13_T)

Sin1AmpRec_Uls_T_f32 = FPM_FixedToFloat_m(MtrPos_EOLDataPtr_Cnt_M_Str->Sin1AmpRec_Uls_u3p13, u3p13_T)

#### Motor Position Measurement

#### Run Time Learning

#### Back EMF Diagnostic

#### Correlation Diagnostic

#### Validity Diagnostic

#### Store Local copy of outputs into Module Outputs

MtrPos2_PrevCorrectedMtrPos_Rev_M_f32 = CorrectedMtrPos_Rev_T_f32

MtrPos2_ErrorTerm_Rev_D_f32 = CorrelationDelta_Rev_T_f32

MtrPosValidErr_VoltsSqrd_D_f32 = ValidErr_VoltsSqrd_T_f32

Rte_IWrite_MtrPos2_Per1_CorrectedMtrPos_Rev_f32(CorrectedMtrPos_Rev_T_f32)

Rte_IWrite_MtrPos2_Per1_SysCCorrectedMtrPos_Rev_f32(CorrectedMtrPos_Rev_T_f32)

Rte_IWrite_MtrPos2_Per1_MechMtrPos_Rev_f32(MechMtrPos_Rev_T_f32)

Rte_IWrite_MtrPos2_Per1_SysCMechMtrPos_Rev_f32(MechMtrPos_Rev_T_f32)

Rte_IWrite_MtrPos2_Per1_CosTheta1_Volt_f32(FPM_FixedToFloat_m(MtrPos_CosTheta1_Volts_G_s2p13, s2p13_T))

Rte_IWrite_MtrPos2_Per1_SinTheta1_Volt_f32(FPM_FixedToFloat_m(MtrPos_SinTheta1_Volts_G_s2p13, s2p13_T))

IdleDblBuf_Cnt_T_u08 = (MtrPos2_LearnedParamBfr_Cnt_M_u08 & 1)^1

MtrPos2_Cos1RTAmpRec_Uls_M_u3p13[IdleDblBuf_Cnt_T_u08] = FPM_FloatToFixed_m(MtrPos2_Cos1RTAmpRec_Uls_M_f32, u3p13_T)

MtrPos2_Cos1RTOffset_Volts_M_u3p13[IdleDblBuf_Cnt_T_u08] = FPM_FloatToFixed_m(MtrPos2_Cos1RTOffset_Volt_M_f32, u3p13_T)

MtrPos2_Sin1RTAmpRec_Uls_M_u3p13[IdleDblBuf_Cnt_T_u08] = FPM_FloatToFixed_m(MtrPos2_Sin1RTAmpRec_Uls_M_f32, u3p13_T)

MtrPos2_Sin1RTOffset_Volts_M_u3p13[IdleDblBuf_Cnt_T_u08] = FPM_FloatToFixed_m(MtrPos2_Sin1RTOffset_Volt_M_f32, u3p13_T)

MtrPos2_LearnedParamBfr_Cnt_M_u08 = IdleDblBuf_Cnt_T_u08

#### Program Flow End

Rte_Call_MtrPos2_Per1_CP1_CheckpointReached()

## Fault Recovery Functions

None

## Shutdown Functions

None

## Interrupt Functions

None

## Serial Communication Functions

### SCom: MtrPos2_SCom_ReadEOLMtrCals

| Arguments Passed | Type |
| --- | --- |
| MtrCalDataPtr | MtrPosCal_DataType * |

#### Design Rationale

None

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Processing of function

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

### SCom: MtrPos2_SCom_SetEOLMtrCals

| Arguments Passed | Type |
| --- | --- |
| MtrCalDataPtr | MtrPosCal_DataType * |

#### Design Rationale

None

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Processing of function

#### Store Local copy of outputs into Module Outputs

Rte_IWrite_MtrPos2_SCom_SetEOLMtrCals_EOLBEMF_Rev_f32(FPM_FixedToFloat_m(Rte_Pim_MtrPosSnsr_EOLData()->BEMFCal_Rev_u0p16, u0p16_T))

#### Program Flow End

N/A

# Execution Requirements

## Execution Sequence of the Module

See section 6.3.1.1 for details on task ordering.

## Execution Rates for sub-modules called by the Scheduler

This table serves as reference for the Scheduler design

| Function Name | Calling Frequency | System State(s) in which the function is called |
| --- | --- | --- |
| MtrPos2_Init1 | On Event | On Init |
| MtrPos2_Per1 | 2 ms | ALL |

## Execution Requirements for Serial Communication Functions

| Function Name | Sub-Module called by (Serial Comm Function Name) |
| --- | --- |
| MtrPos2_SCom_ReadEOLMtrCals |  |
| MtrPos2_SCom_SetEOLMtrCals |  |

# Memory Map Definition Requirements

## Sub Modules (Functions)

This table identifies the software segments for functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| MtrPos2_Init1 | RTE_START_SEC_SA_MTRPOS2_APPL_CODE |
| MtrPos2_Per1 | RTE_START_SEC_SA_MTRPOS2_APPL_CODE |

## Local Functions

This table identifies the software segments for local functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| OffsetInit |  |
| RunTimeFilter |  |
| CalcOffsetFromMinMax |  |
| CalcSinAmpRec |  |
| CalcCosAmpRec |  |

# Known Issues / Limitations With Design

- INLINE functions defined in GlobalMacro.h are not unit tested.

# Revision Control Log

| Item # | Rev # | Change Description | Date | Author Initials |
| --- | --- | --- | --- | --- |
| 1 | 1.0 | Initial Version | 23-Oct-12 | OT |
| 2 | 2.0 | UTP updates | 18-Dec-12 | OT |
| 3 | 3.0 | MDD Catchup for SRC Ver 4 | 13-June-13 | NRAR |